SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0713

Name: Y Guru Charan

Reg No: 192565020

COURSE OUTCOME COVERED

CO5:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

Assessment Tool Weightage: 40%

QUESTIONS: 3 Total Marks:30

Annexure A

Constraint-Based Problem Solving

Code of Conduct:

I Y Guru Charan (192565020) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Y Guru Charan

Constraint-Based Problem Solving

1. A multinational company is opening a new branch office in a remote

rural location where high-speed wired internet is unavailable. The office

must support cloud-based applications, video conferencing, and secure

communication with the headquarters. The company has a limited budget

for connectivity but requires uninterrupted business operations. As a

network engineer, recommend a suitable internet connection, networking

devices, and security mechanisms. Justify your recommendations by

considering bandwidth limitations, reliability, cost, and future scalability

Answer:

For the rural branch office, a hybrid network topology using 4G/5G wireless broadband as the primary connection and satellite Internet as backup is suitable.

Proposed Design

Internet → 4G/5G Router → Firewall → Managed Switch → Wi-Fi 6 APs / PCs

The 4G/5G connection provides the main Internet access because it is cheaper and easier to deploy than wired broadband. A satellite connection can be used as a backup to maintain business operations when the cellular connection fails.

Networking Devices

- 4G/5G Router: Provides Internet connectivity.

- Dual-WAN Router: Automatically switches to the backup connection.

- Firewall: Filters unauthorized traffic.

- Managed Gigabit Switch: Connects computers, printers, servers, and access points.

- Wi-Fi 6 Access Points: Provides wireless connectivity to employees.

- VPN Gateway: Creates a secure connection with headquarters.

Security Mechanisms

- Site-to-site IPsec VPN for headquarters communication.

- Firewall with IDS/IPS to detect and block attacks.

- WPA3 for secure Wi-Fi.

- VLANs to separate employees, guests, and critical systems.

- Strong authentication and MFA for sensitive services.

2. A university plans to provide campus-wide wireless internet access for

more than 3,000 students across classrooms, laboratories, libraries, and

hostels. The network must support a large number of simultaneous users

while minimizing signal interference and ensuring secure access. Due to

budget constraints, only a limited number of wireless access points can be

deployed. Analyze the requirements and propose an efficient wireless

network design, including access point placement, frequency band

selection, and authentication methods. Explain how your solution

balances coverage, performance, security, and cost.

Answer:

For more than 3,000 students, a centrally managed wireless network using a hybrid/star topology is recommended. Access points should be strategically placed in classrooms, laboratories, libraries, and hostels.

Proposed Design

Core Switch/Controller

→ Classroom APs
→ Laboratory APs
→ Library APs
→ Hostel APs

Each AP connects to the network through Cat6/Cat6A Ethernet cabling using PoE.

Access Point Placement

- Install APs in high-user-density areas.

- Place APs near classrooms and lecture halls.

- Use additional APs in libraries and hostels where user density is high.

- Avoid placing APs near thick concrete walls and sources of interference.

- Perform a site survey before installation.

- Use lower transmit power where necessary to reduce overlapping coverage.

Frequency Selection

- 2.4 GHz: Used mainly for longer-range coverage and older devices.

- 5 GHz: Preferred for high-speed applications because it provides more channels and less interference.

- 6 GHz: Can be used where supported by Wi-Fi 6E/7 devices for additional capacity.

Authentication and Security

- WPA3-Enterprise for secure student access.

- 802.1X authentication with a RADIUS server.

- Separate VLANs for students, staff, guests, and administration.

- Captive portal can be used for guest access.

3. A financial institution is upgrading its internal computer network to

improve cybersecurity while allowing employees to access sensitive

customer information and online banking services. The organization must

comply with strict security policies without significantly affecting

network performance or increasing operational costs. Design a secure

network architecture by selecting appropriate network segmentation

techniques, firewall deployment, authentication methods, and intrusion

detection mechanisms. Justify your design by considering the constraints

of security, performance, compliance, and budget.

Answer:

A financial institution requires strong security because it handles sensitive customer and banking information. A segmented hybrid network architecture with firewalls, VLANs, authentication, and IDS/IPS should be implemented.

Proposed Architecture

Internet

↓

Edge Router

↓

Perimeter Firewall

↓

DMZ

- Public Web Server

- Online Banking Server

↓

Internal Firewall

↓

Core Layer 3 Switch

→ Employee VLAN

→ Customer Data VLAN

→ Server VLAN

→ Management VLAN

→ Guest VLAN

Network Segmentation

VLANs should separate different types of users and systems.

- VLAN 10: Employees

- VLAN 20: Customer database systems

- VLAN 30: Application/Banking servers

- VLAN 40: Network management

- VLAN 50: Guest users

Access Control Lists (ACLs) restrict communication between VLANs.

Firewall Deployment

- Perimeter firewall: Protects the organization from Internet-based attacks.

- Internal firewall: Controls traffic between sensitive network segments.

- Web Application Firewall: Protects online banking applications from web-based attacks.

Authentication

- 802.1X for network access.

- MFA for sensitive systems.

- Role-Based Access Control (RBAC).

- Strong password policies.

Intrusion Detection

An IDS/IPS continuously monitors network traffic.

- IDS detects suspicious activity.

- IPS automatically blocks known malicious traffic.

- Logs are sent to a centralized monitoring/SIEM system.